## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Drives (4115)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Electrical drive & draw its block diagram. |  |
| b) | Classify load torque and gives any two points of distinction between the types of load torques. |  |
| c) | State the purposes of electrical braking, giving an example of drive serving each purpose. |  |
| d) | State the expression for temperature rise in electric motor and indicate each term used in expression. |  |
| e) | Give two points of distinction between overload relay and time delay relay. |  |
| f) | State the types of motors used in cranes and justify your choice. |  |
| g) | State the types of drives used in paper mills and textile mills. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a four quadrantal diagram to indicate the operation of motor used in electric drive of your choice. Also indicate on it the following (i) speed of rotation of motor and its direction (ii) Load torque and motor torque direction. Name each quadrant and explain its operation in brief. |  |
| b) | State the factors on which variation of load torque of drive depends. Also indicate one example of drive satisfying each of such factor of variation of load torque and draw a characteristic of variation of load torque w.r.t. such factors. |  |
| c) | Define (i) Stability (ii) Steady state stability (iii) Instability in electric drive. |  |
| d) | A weight of 1000 Kg is to be lifted up at a velocity of 1m/sec by means of a winch having a diameter of 0.3m. The winch is driven by a motor running at 960 rpm. Determine load torque of the system referred to motor shaft. The motor driving a winch is as shown in figure A. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Is regenerative braking applicable for d.c. series motor. Give your choice & justify it. Also state the types of brakings applicable for d.c. series motor and draw speed v/s torque characteristic of any one of such braking of your choice & explain it in brief. |  |
| b) | State the types of dynamic brakings we came across in A.C. 3 phase Induction motor. Explain in brief any one of such type of dynamic braking of your choice with the help of neat circuit diagram and speed v/s torque characteristic. |  |
| c) | Explain in brief with the help of neat diagram the method of injection of an external e.m.f into the rotor circuit. For which motor is it applicable and what purpose does it serve? |  |
| d) | Explain in brief pole changing method for speed control of induction motor for four pole and eight pole arrangement on stator, with the help of neat winding connection diagrams. | …….2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List out the different types of enclosures we use for electric motor. What are their functions? Also give one application of each type of enclosure used. |  |
| b) | What is load fluctuation? What are its causes? What are the effects of continuous load fluctuations? How can such fluctuations be minimized? Explain the method used in brief. |  |
| c) | What is the effect of load inertia on selection of rating of motor? Explain it in brief. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the necessity of interlocking in operation of motor? Draw a neat labelled control circuit diagram to indicate the interlocking operation in a motor and explain it in brief. |  |
| b) | State the methods used for reversal of direction of rotation of motor. Also draw a neat labelled control circuit diagram to indicate operation of motor fed from two sources of supply. Also draw power circuit diagram for the same. |  |
| c) | Draw the neat labelled control circuit diagram to indicate remote control operation in Induction motor & remote control operation in Induction motor & explain it in brief. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the types of motors used for following applications & justify their choice: (i) Refrigeration & air-conditioning (ii) Lifts. |  |
| b) | Give two points of distinction between: - Limit switch & selector switch - Two types of push button switches - Proximity switch & Float switch |  |
| c) | State the methods used in rotor resistance control. For which motor are they applicable? What purpose does it serve? Explain in brief, with the help of neat circuit diagram, Rotor resistance control using chopper. |  |