## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Civil Engg./CT

Subject: Analysis of Structures(4026)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Derive an expression for fixed end moments for a fixed beam of span ‘L’ carrying a point load ‘W’ at mid span. |  |
| b) | A flitched beam consists of a timber section with steel plates on its top and bottom edges. If  is the diameter of screws, explain how do you determine the pitch of screws. |  |
| c) | What is shear connector? Stating its necessity, explain the procedure involved in its design. |  |
| d) | Considering an example, explain how the Mohr’s circle is used in solving problems of combined stresses. |  |
| e) | A Simple supported beam of span ‘L’ is subjected to equal loads (W/2) at each of its 1/3 span. Find the deflection under each load. Take EI constant for the beam. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The cross section of a composite beam consists a solid steel rod 160mm diameter encased in a bronze sleeve of 30mm thickness, shrunk on the core so that there is no slipping at the common surface. Determine the moment of resistance of this composite section, if the maximum permissible skin stresses in steel and bronze are 100N/mm2 and 60N/mm2 respectively. Take Es=2 Eb. |  |
| b) | A masonry dam 10m high, 2m wide at top and 5m wide at the base has its water face vertical and retains water to a depth of 7.5m. Calculate the maximum and minimum stresses induced at the base, if the unit of masonry and water are 23KN/m3 and 10KN/m3 respectively. |  |
| c) | A short hollow C.I cylindrical column is 250mm external diameter and 20mm internal diameter. In casting, the bore got eccentric so that the thickness varies from 20mm at one end and 30mm at the other end. If the column carries a load of 750KN along the axis of bore, calculate the extreme stresses induced in the section. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | At a point in a strained material there is tensile stress of 90N/mm2 upon horizontal plane and a compressive stress of 50N/mm2 upon vertical plane. Also there is shear stress of 60N/mm2 upon each of these planes. Determine the planes of maximum shear stress at the point and also resultant stress on the planes of maximum shear stress. |  |
| b) | The stresses at a point in a component are 100N/mm2 (compressive) and 60N/mm2 (tensile). Determine the magnitude of normal and shear stresses on plane inclined at an angle of 30 with compressive stress. Also determine the direction of resultant stress and magnitude of maximum shear stress intensity. Use Mohr’s circle method. |  |
|  |  | …….2/- |
|  | -2- |  |
| c) | In a material subjected to strain, the resultant stress across a certain plane is 75N/mm2 tensile, inclined at 30 to its normal inducing clock wise shear on the plane. The normal stress across the plane at right angles to this is 45N/mm2, tensile. Find the principal stresses and locate their planes. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the slope and deflation at the free end and at a point 1.5m from the fixed end of a cantilever beam having 5m span. The loads acting at free end and at 1.5m and 3m are 8KN, 12KN and 15KN respectively. Take E = 2x105 N/mm2, I = 2.5 x 108mm4. |  |
| b) | A beam simply supported over an effective span of 8m carries concentrated loads of 45KN, 25KN, and 50KN at 1.5m, 5m and 6.5m from the left end support. Using Macauly’s method, determine the position and magnitude of maximum deflection. Take EI = 8x1012 N-mm2. |  |
| c) | A fixed beam of span 7.5m carries a point load of 50KN at 2.5m from left support. If is also subjected to a moment of 60KN-m (Clockwise) at a point 5m from left support. Calculate support moments and draw SFD and BMD. |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | Using moment distribution method analyse a continuous beam ABCD. Span AB=5m, BC=4m, CD=4.5m. End ‘A’ is fixed while ‘D’ is hinged. Span AB carries udl of 25KN/m, BC carries udl of 20KN/m and span CD carries a point load of 40KN at a distance of 2m from support ‘C’. Take MI constant for all spans. | (8) |
|  | ### OR |  |
| a) | For a portal frame ABCD with supports ‘A’ and ‘D’ hinged carries an udl of 25KN/m over the entire span BC=4.5m. Height of each column AB and CD are 4m. Flexural rigidity for beams is 1.5 EI and for columns 2EI each. Find support moments and draw BMD for portal frame. Use moment distribution method. | (8) |
| b) | Describe the following: i) Stiffness ii) Carryout factor iii) Distribution factor. | (4) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A hollow cylinder cast iron column 5m long, both ends being fixed. Design the column to carry an axial load of 300KN. Use Rankine’s formula and adopt a factor of safety of 5. The internal diameter may be taken as 0.85 times the external diameter. Take c = 550N/mm2 and  = 1/1600. |  |
| b) | An RS joist of length 4.5m and cross sectional dimension of 300mm x 200mm has both its end fixed. Calculate the crippling load by Rankine’s formula. Take  = 325N/mm2,  = 1/7500. |  |
| c) | A steel stanchian consists of two channels, ISMC 350 @ 413N/m placed back to back spacing of 225mm. Find the crippling load for the column, if its length is 6m with both ends fixed. Take E = 2 x 105N/mm2. Use Euler’s formula. Properties of one channel section IXX = 10008 x 104mm4 IYY = 430.6 x 106mm4 A = 5366mm2 overall depth = 350mm CYY = 24.4mm |  |

***********