## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg.

Subject: Principles of Refrigeration(5046)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define refrigeration. State at least five methods of refrigeration. |  |
| b) | Sketch T-S diagram for Reversed carnot cycle. Name the processes in Reversed carnot cycle. |  |
| c) | In a Vapour Refrigeration system, the refrigerant enters the compressor in dry saturated state and leaves the condenser in sub-cooled state. Draw P-H and T-S diagram for the above cycle. |  |
| d) | Differentiate between primary refrigerant and secondary refrigerant. Name at least two refrigerants of each type. |  |
| e) | State at least three points of difference between Dry type Evaporate and Flooded type Evaporator. |  |
| f) | What are the probable causes of flow of over current in the motor of compressor? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the construction and working principles of Vapour compression refrigeration system. |  |
| b) | What are the advantages of super heating and sub-cooling? How it is achieved? |  |
| c) | An ammonia refrigerator works between –8C and 26C. Assuming simple saturation cycle, determine the following: - Temperature of vapour at the end of compression - Carnot C.O.P. - Theoretical C.O.P. - Refrigeration effect in TR if 0.5 kg of refrigerant is circulated per second. |  |
|  | Use the following properties of Ammonia. Saturation Temperature in C; Enthalpy in; Entropy in. KJ/Kg; KJ/Kg; KJ/Kg-K. hf; hg; sg. -8; 163.18; 1452.81; 5.76. 26; 322.73; 1483.72; 5.31. |  |
|  | Specify heat of Ammonia vapour Cpv = 3.158 KJ/Kg-K |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the designation number for the following refrigerants: - Mono-chloro Di-flouro methane - Tetra – flouro ethane. |  |
| b) | State at least eight desirable properties of an ideal refrigerant. (primary) |  |
| c) | State at least two functions of following components in vapour absorption cycle: i) Absorber ii) Generator iii) Analyser iv) Rectifier. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the construction and working principle of ‘Shell and Tube’ type condenser. |  |
| b) | With a neat sketch explain the construction and working of ‘Thermostatic Expansion Valve’. |  |
| c) | What is de-frosting? What are the effects of frost on the performance of refrigeration system? List at least six methods of De-frosting. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the probable causes and effects of low-pressure on the refrigeration system? |  |
| b) | With a neat sketch explain the construction and working of ‘High Pressure Cut-outs’. |  |
| c) | State at least eight points of difference between Vapour compression and Vapour absorption refrigeration system. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled diagram of ‘Domestic Refrigerator’. |  |
| b) | Write a note on low temperature applications of refrigeration system. |  |
| c) | Write a note on ‘Role of refrigeration in food preservation’. |  |

***********