## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical Engg.

Subject: Circuit Theory(EE301) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Unilateral circuit ii) Passive network. |  |
| b) | State and explain Kirchoff’s Voltage Law or Mesh Law. |  |
| c) | Define i) Power factor ii) Reactive power. |  |
| d) | What is Bandwidth of Resonant circuits? |  |
| e) | A balanced 3, 3 wire system with Y connected load for which voltage is 230V and impedance of each phase is (6 + j 8) ohm. Find line current. |  |
| f) | When are transient disturbances produced in a circuit? |  |
| g) | What is Q factor of a parallel AC circuit? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In the fig. 2(a) find the current flowing in the 4 Resistor using the principle of superposition. |  |
| b) | In the fig. 2(b) frame and solve the node equations of the network. Hence find the total power consumed by the passive elements of the Network. |  |
| c) | Using Nortons theorem find the current that would flow through the Resistor R2= 12. Refer fig. 2(c) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A coil having a resistance of 15 ohm and an inductance of 0.2H is connected in series with another coil having a resistance of 25 ohm and an inductance of 0.04H to a 230V, 50 HZ supply. Determine i) the voltage across each coil ii) power dissipated in each coil iii) power factor of the circuit as a whole. |  |
| b) | An ac voltage (80+j60)V is applied to a circuit and the current flowing is (-4+j10) Amperes. Find i) Impedance of the circuit ii) power consumed and iii) phase angle. |  |
| c) | Two circuits the impedances of which are given by Z1 = (10+j15) and Z2 = (6-j8) ohm are connected in parallel. If the total current supplied is 15A, what is the power taken by each branch. Find also the power factor of Individual circuits and of combination. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A balanced star connected load of (8+j6) per phase is connected to a balanced 3 phase 400V supply. Find the line current, power factor, power and total volt amperes. |  |
| b) | Three impedances each of magnitude (15-j20) are connected in mesh across a 3 phase, 400 volts ac supply. Determine the phase current, line current, active power, reactive power drawn from supply. |  |
| c) | Derive the relationship between line and phase quantities in a star connected system. |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An R-L-C series circuit consists of a resistance of 1000, an inductance of 100mH and a capacitance of 1x10-12 Farads. If a voltage of 100V is applied across the combination, find i) Resonance frequency ii) Q factor of the circuit iii) half power points. |  |
| b) | A coil of 10 ohm resistance and 0.1H inductance is connected in parallel with a capacitor of 100 μF capacitance. Calculate the frequency at which the circuit will act as a non inductive resistance of ‘R’ ohm. Find also the value of dynamic resistance. |  |
| c) | Derive the expression for resonant frequency in a parallel AC circuit. |  |
| d) | The four arms of a Hay’s bridge are arranges as follows: AD is a coil of unknown impedance; DC is a non inductive resistance of 880 in series with a standard capacitor of 1.4 μF, BA is a non inductive resistor of 16,500. If the supply frequency is 50Hz and the bridge is balanced. Find the inductance and resistance of the coil. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In a 3 phase 4 wire system the currents in the R, Y and B lines under abnormal conditions of loading were as follows: IR= 10030 IY= 50300 IB=30180 Calculate the positive, negative and zero phase sequence currents in the R-line and the return current in the neutral conductor. |  |
| b) | The four arms of a Wein parallel bridge ABCD supplied with a sinusoidal voltage have the following values: AB- 200 ohm resistance in parallel with 1 μF capacitor BC- 400 ohm resistance ; CD- 1000 ohm resistance and DA - resistance R in series with a 2 μF capacitor. Determine i) The value of R and ii) Supply frequency at which the bridge will be balanced. |  |
| c) | Find out the positive, negative and zero phase sequence components of the following set of three unbalanced voltage vectors: VA = 15145 VB= 1030 and VC=30-60 |  |
| d) | Three impedances Z1= (3+j4) ; Z2 = (2+j3.46)  and Z3 = (1-j7.46) are connected in series. Refer fig. 6(c). The voltage drop across Z1 is (10+j0) volts. Find i) current in the circuit ii) voltage drop across Z2 and Z3 iii) Voltage of the supply. |  |