## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical Engg.

Subject: Distribution & Estimation(4108)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the terms Feeder, Distributor & Service mains. |  |
| b) | State any 3 I.E. rules relevant to earthing. |  |
| c) | What is cleat wiring system? Where it is used? |  |
| d) | List out standard conductors used for rural electrification as per REC. |  |
| e) | What is the frequency of occurrence of symmetrical & unsymmetrical faults in distribution system? |  |
| f) | What is earthing? What is the purpose of doing it? |  |
| g) | Give any 3 points of comparison between surface conduit & concealed conduit wiring. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare between Radial, Ring & Interconnected systems on the basis of load, reliability of supply & economy. |  |
| b) | Define & explain the importance of following terms: (i) Connected load (ii) Demand factor (iii) Diversity factor. |  |
| c) | List the components of a distribution line & state function of each. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain SWER wiring system with its advantages & disadvantages. |  |
| b) | A 1, 2 wire feeder, 1500m long, supplies a load of 60A at 0.8 p.f. & 50A at 0.88 p.f. at a distance of 600m, 1500m respectively from the feeding point. The resistance & reactance of the feeder per km length are 0.06 & 0.1 respectively. If voltage at far end is to be maintained at 220V, calculate the voltage at sending send. |  |
| c) | Where will you use the following types of guy & why? (i) Wind guy (ii) Angle guy (iii) Bow guy (iv) Strut guy. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Estimate the main material required for a 750m 415/240V, 3- 4 wire system in vertical configuration. The line starts from a substation to feed a load of 30KW. Take span between two poles as 50m. The size of conductor is ACSR 6/1 x 2.59 mm (code weasel) & weighs 128 Kg/Km. |  |
| b) | Explain the various types of faults in distribution system & their effects on system. |  |
| c) | With a neat diagram explain the following: (i) Stair-case wiring (ii) Godown wiring. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A house is to be wired with PVC casing & capping wiring system consisting of following points: - Light points – 20 of 60W each - Fan points – 6 of 60W each - Wall socket points – 5 of 60W each - Power points – 5 of 1000W each. Determine: (i) Number of light & power subcircuits (ii) Size of main switch & distribution box. |  |
| b) | In a workshop of size 25m x 10m, a 10 HP, 400V, 3 I.M. is to be installed for wood cutting. (i) Draw a wiring diagram (ii) Find rating of main switch & motor switch (iii) Find size of wire for the motor. |  |
| c) | With a neat diagram explain construction & working of ELCB. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | With a neat sketch explain plate earthing. |  |
| b) | Briefly explain factors affecting earth resistance. |  |
| c) | Calculate loop inductance per km of a 1 transmission line comprising of 2 parallel conductors 1m apart having 1.25 cm diameter. Also calculate line reactance per km at line frequency of 50Hz. |  |
| d) | Write a short note on determination of rating of fuses, size of conduit, main switch & starter for installation of motors. |  |

***********