## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Civil Engg./CT/M

Subject: Hydraulics(4025)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the properties of Real fluid? |  |
| b) | What are the types of fluid flow? |  |
| c) | Explain total pressure and centre of pressure. |  |
| d) | Define (i) coefficient of velocity (ii) coefficient of discharge (iii) Coefficient of contraction. |  |
| e) | Define (i) Notch and (ii) Weir. |  |
| f) | State Pascal’s law. |  |
| g) | Define (i) Viscosity (ii) Absolute pressure. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A tube of monometer is used to measure pressure of oil of specific gravity 0.85 flowing in a pipeline. Its left limb is connected to pipe and right limb is open to atmosphere. The centre of pipe is 100mm below level of mercury in the right limb. If the difference of mercury level in two limbs is 160mm. Find the absolute pressure in KPa. |  |
| b) | A horizontal venturimeter 160mm x 80mm is used to measure flow of an oil of specific gravity 0.3. Determine the deflection of oil mercury guage if discharge of oil is 50 lps. Take coefficient of discharge as 1. |  |
| c) | A square plate with 1m side is vertically immersed in a liquid of specific gravity 0.85. A corner of the square plate is at a distance of 1.5m below the free surface and the diagonal containing the corner is normal to the free surface. Find (i) Total pressure and (ii) Centre of pressure. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | On the suction side of a pump a guage shows a negative pressure of 0.35 bar. Express this pressure in terms of: - Intensity of pressure KPa - N/m2 absolute - Metres of water guage - Metres of oil (specific gravity 0.82) absolute - Centimetres of mercury guage. Take atmosphere pressure as 76 cm of Hg and relative density of mercury as 13.6. |  |
| b) | Water is flowing through a pipe having diametres 600mm and 400mm at the bottom and upper end respectively. The intensity of pressure at the bottom end is 350 KN/m2 and the pressure at the upper end is 100 KN/m2. Determine the difference in datum head if the rate of flow through the pipe is 60 l/s |  |
| c) | A rectangular channel is to be dug in the rocky portion of a soil. Find its most economical cross-section if it is to convey 12 m3/s of water with an average velocity of 3m/s. Take Chezy’s constant as 50. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What is meant by most economical section of a channel? State the conditions for the trapezoidal section to be most economical. |  |
| b) | Write brief note on: Hydraulic Intensifier. |  |
| c) | Explain the working of a Reciprocating pump. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A tank has two identical orifices in one of its vertical sides. The upper orifice is 2m below the water surface and lower one is 3m below the water surface. Find the point at which the two jets will intersect, if the coefficient of velocity is 0.92 for both the orifices. |  |
| b) | The head of water over a rectangular weir is 400mm. The length of the crest of the weir with end contraction suppressed is 1.5m. Find discharge using the following formulae: (i) Francis Formula (ii) Bazin’s Formula. |  |
| c) | At a sudden enlargement of a water main from 250mm to 450mm diameter, the hydraulic gradient rises by 10mm. Calculate the rate of flow. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A piping system consists of three pipes arranged in series; the lengths of the pipes are 1200m, 750m and 500m and diameters are 700mm, 600mm and 400mm respectively. Determine an equivalent diameter for the pipe 3000m long. |  |
| b) | Write a short note on ‘Hydraulic Jump’. |  |
| c) | Write short note on (i) Hydraulic Accumulator OR (ii) Water hammer in pipes. |  |

***********