## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Estimation & Costing(4242)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the different functions of Estimating department. |  |
| b) | Explain the concept of Estimating. |  |
| c) | Explain in short different sheet metal joints. |  |
| d) | Write a short note on “Beyond Economical Repairs”. |  |
| e) | Explain the factors affecting welding cost. |  |
| f) | Define Item Rate Estimate. State its advantages over type of estimate. |  |
| g) | Differentiate between Revised Estimate and Supplementary Estimate. |  |
| Q.No.2. | Prepare a Measurement sheet as well as Abstract sheet for “Truss Bridge Structure” of span 15m for the following items of work: i) Structural steel ii) Painting. Number of trusses are two. Assume suitable market rates for the items. Refer Fig.1. | (12) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Calculate the time required to machine the job shown in fig.2 of mild steel from a 50mm diameter round stock of length 100mm. Assume suitable machining parameters. | (8) |
| b) | Explain the reasons for replacement of equipments. | (4) |
|  | ### OR |  |
| b) | Define Cutting speed, Feed and Depth of cut in machining. | (4) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A company produces 3500 items per month. The fixed cost and variable cost of the company are Rs.1,50,000/- and Rs.3,00,000/- per month. Determine the cost of the product. If the production is increased to 4200 items per month, then what should be the new selling price of the product at a constant profit of Rs. 80,000/- per month? |  |

……2/-

-2-

| b) | Explain different sheet metal operations. |  |
| --- | --- | --- |
| c) | Calculate the labour cost of a arc welded joint per metre if the material deposition rate is 2Kg/hr on a single bevel butt joint of 10mm thick plates and 60 included angle. The diameter of M.S. electrodes is 4mm and its length is 450mm. The cost of single electrode is Rs. 4.5/- and labour is paid Rs. 350/- per day. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A machine was purchased for Rs.56,000/- on 1st June 1988. The installation cost was Rs.8,000/-. Machine was to be replaced by a new one on 31st May 2003. If the scrap value was estimated as Rs.16,000/-, what should be the rate of depreciation and depreciation fund on 15th December 1998? If after 11 years of running some machine parts are replaced and the replacement cost is Rs. 1,800/-, what will be the new rate of depreciation? |  |
| b) | What are the objectives of costing? |  |
| c) | Estimate the cost of material of the job shown in fig.2 made up of mild steel costing Rs. 210/Kg and having specific weight of 7.85 gms/cc. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define a Tender notice. Also draft a Tender notice given by Superintend Engineer of PWD for construction of Tower for Rs. 0.5 crores. |  |
| b) | Explain the following in contracts: i) Arbitration ii) Work Charged Establishment iii) Schedule of rates. |  |
| c) | - State the conditions of a valid contract. - State the requirements of contract document. |  |