## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg./A

Subject: Theory of Machines(4039)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between machine and structure? |  |
| b) | Draw a crank effort diagram for 4 stroke single cylinder I.C. engine? |  |
| c) | What is the function of Governor? How does it differ from that of flywheel? |  |
| d) | With suitable sketches compare open & cross belt drive? |  |
| e) | Draw a neat sketch and show for gear i) Top land ii) Flank iii) Circular pitch. |  |
| f) | What is wheel balancing & how is it done? |  |
| g) | The engine of an aeroplane rotates in clockwise direction when seen from the tail end and the aeroplane takes left turn. What is the effect of the gyroscopic couple on the aeroplane? |  |
| h) | How are the dynamometers different from the brakes? |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | The dimensions of the various links of a mechanism as shown in fig.1 are: OA=80mm, AC=CB=CD=120mm. If the crank OA rotates at 150 r.p.m in the anti-clockwise direction, find for the given configuration (i) Velocity and acceleration of B & D (ii) Angular acceleration of the links AB & CD. | (6) |
|  | ### OR |  |
| a) | In a slider crank mechanism, the length of the crank and connecting rod are 100mm and 400mm respectively. The crank rotates uniformly at 600 r.p.m clockwise. Using Klien’s construction method find (i) Velocity and acceleration of Slider (ii) Angular velocity and angular acceleration of connecting rod when crank has turned through 45 from I.D.C. | (6) |

…….2/-

-2-

| b) | A petrol engine has a stroke of 120mm & connecting rod is 3 times the crank length. The crank rotates at 1500 r.p.m in clockwise direction. Using analytical method determine: (i) Velocity and acceleration of piston (ii) Angular velocity and angular acceleration of connecting rod when piston has travelled one fourth of its stroke from IDC. | (6) |
| --- | --- | --- |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain the construction and working of Wilson Hartnell governor. |  |
| b) | A vertical shaft supports a load of 20 KN in a conical pivot bearing. The external radius of the cone is 3 times the internal radius and the cone angle is 120. If uniform intensity of pressure is 0.35 MN/m2, determine the dimensions of bearing. If the coefficient of friction between the shaft and the bearing is 0.05 and the shaft rotates at 120 rpm, find the power absorbed in friction. |  |
| c) | The thrust on the propeller shaft of a marine engine is taken up by 8 collars whose external and internal diameters are 760 mm and 450 mm respectively. The thrust pressure is 0.5 MPa and may be assumed uniform. The coefficient of friction between the shaft and collars is 0.04. If the shaft rotates at 90 r.p.m, find (i) Total thrust on collars (ii) Power absorbed by friction in bearings. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A shaft which rotates at a constant speed of 150 r.p.m is connected by belting to a parallel shaft 720mm apart, which has to run at 70, 90 & 110 r.p.m. The smallest pulley on the driving shaft is 50mm in radius. Determine remaining radii of the two stepped pulleys for (i) Crossed belt drive (ii) Open belt drive. |  |
| b) | A flat belt, 8mm thick & 100 mm wide transmits power between two pulleys, running at 1600 m/min. The mass of the belt is 0.9 Kg/m length. The angle of lap in the smaller pulley is 165 and the coefficient of friction between the belt and pulley is 0.3. If maximum permissible stress in belt is 2 MN/m2, find (i) Maximum power transmitted (ii) Initial tension in the belt. |  |
| c) | With the help of a neat sketch, explain the construction and working of a Whithworth Quick Return motion mechanism. |  |

……..3/-

-3-

| Q.No.5. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | A disc cam rotating in clockwise diretion is used to move knife edge reciprocating follower as below: - Outstroke through 40mm during 110 of cam rotation with equal and uniform acceleration and retardation. - Dwell for the next 60 of cam rotation. - Return stroke with uniform velocity through 40mm during next 80 of cam rotation. - Dwell during remaining portion of cam rotation. The line of stroke of the follower is 15mm offset on right side of cam shaft axis. The minimum radius of cam is 50mm. Draw the profile of the cam. If cam rotates at a uniform speed of 300 r.p.m, find the maximum velocity and acceleration of follower during return stroke. | (8) |
| b) | A compound gear train consists of six gears. The number of teeth on the gear area as follows. Gear; A; B; C; D; E; F. Number of Teeth; 80; 50; 60; 30; 40; 24. The gears B and C are on one shaft while gears D & E on another shaft. The gear A drives gear B, gear C drives gear D and gear E drives gear F. If the gear A rotates at 120 r.p.m, find the speed of gear F and train value of the gear train. | (4) |
|  | ### OR |  |
| b) | State ‘Law of gears’. Define i) Pressure angle ii) Module for gears. | (4) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A differential band brake shown in fig.2 has the diameter of the drum 800mm. The coefficient of friction between band and the drum is 0.3. The angle of contact between band & drum is 240. When the force 600N is applied at the free end of lever, find for clockwise rotation of the drum (i) the maximum & minimum forces in the band (ii) the torque which can be applied by the brake. |  |
| b) | Explain with neat sketches the Bevis-Gibson flash light dynamometer. |  |
| c) | Four masses A, B, C and D are attached to a shaft and revolve in the same plane. The masses are 12 kg, 10 kg, 18 kg and 15 kg respectively and their radii of rotation are 40mm, 50mm, 60mm and 30mm. The angular position of the masses B, C and D are 60, 135 and 270 from the mass A. Find the magnitude and position of the balancing mass at a radius of 100mm. |  |
|  |  | ……4/- |