## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Structural Drawing(4341)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is compulsory. Answer any five from the remaining

Questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Weight curve |  |
| b) | Sagging and hogging |  |
| c) | Keel plating |  |
| d) | Structural continuity |  |
| Q.No.2. | Sketch the midship section of a bulk carrier showing all structural members. | (12) |
| Q.No.3. | Sketch a typical shell expansion plan. Also show how the numbering of plates is done. List the use of shell expansion plan. | (12) |
| Q.No.4. | A box shaped vessel 100m long, floats on an even keel displacing 2000t. The mass of the vessel alone is 1000t evenly distributed and she is loaded at each end for the length of 25m with 500t of cargo also evenly distributed. Sketch the curve of load, shear force and bending moment. State the maximum shear force and Bending moment and where it occurs. | (12) |
| Q.No.5. | Explain the process of determination of sizes of plates and stiffeners in hull structure. | (12) |
| Q.No.6. | With the help of typical sketch explain the procedure to calculate the section modulus of the midship section. Write the bending equation and define all the terms used in the formula. | (12) |
| Q.No.7. | Calculate the following for a midship section shown below: - The positing of neutral axis. - Moment of inertia of the total area of the material about the neutral axis. - Section modulus at the deck and keel. | (12) |

***********