## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-II(4041)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you mean by ‘Critical ratio’ in the nozzle? Explain its significance. |  |
| b) | What do you mean by ‘compounding’ in steam turbines? Why is it required? |  |
| c) | Give any three points of differences between SI & CI engines. |  |
| d) | Explain in brief the working of ‘induced air cooling tower’. |  |
| e) | Sketch the reverse carnot cycle on P-V and T-S plot clearly stating the processes involved. |  |
| f) | Compare gas turbine with steam turbine. (Give any three points) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List down any six points of differences between the impulse & the reaction turbines. |  |
| b) | What do you mean by governing of a steam turbine? Explain the process of throttle governing giving neat sketch. |  |
| c) | What do you understand by regenerative and reheat cycle? Explain. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and working of a two stroke cycle petrol engine. |  |
| b) | Following detail conforms to the air standard Otto cycle: - Lowest and highest temperature in the cycle=27C and 260C. - Compression ratio =10. Calculate the temperatures at salient points and the air standard thermal efficiency. |  |
| c) | Giving neat sketch explain the valve timing diagram for a four stroke cycle diesel engine. |  |
| d) | Explain the various types of steam nozzles in detail giving neat sketches. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the effect of air leakage on working of a condenser. |  |
| b) | Compare a jet condenser with a surface condenser. |  |
| c) | Explain the construction and working of an ignition system of the internal combustion engine giving a neat sketch. |  |
| d) | With the help of a neat sketch explain the construction and working of a jet condenser. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the simple vapour absorption cycle. |  |
| b) | Giving neat sketch explain the vapour compression cycle. |  |
| c) | Write down any four important points of differences between a gas turbine and an IC engine. |  |
| d) | Giving neat sketch explain the starting system of an IC engine. | ……..2/- |
| Q.No.6. | Write notes on any three: | 3 x 4 = 12 |
| a) | Lubrication system of an IC engine |  |
| b) | Pressure compounding |  |
| c) | Closed cycle gas turbine |  |
| d) | Morse test |  |

.    ***********