## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electronics Engg./EC/COMP/EI/MED/E&EE

Subject: Microprocessors-I(8085)[4142]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write a short note on serial and parallel communication. |  |
| b) | Write a short note on stack. |  |
| c) | Define the terms opcode and operand with examples. |  |
| d) | What do you mean by multiple ending subroutines? |  |
| e) | What are data transfer operations? Explain with the help of an example. |  |
| f) | Differentiate between synchronous and asynchronous data transmission. |  |
| g) | State the purpose of following signals of 8085: (i)IOM (ii) HLDA (iii) READY. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagram, explain the role of ALE signal in demultiplexing the Address bus. |  |
| b) | Explain three main functions performed by microprocessor 8085. |  |
| c) | Draw and explain the internal architecture of 8085. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with an example the steps involved in the 8085 fetching an instruction code from a memory location. Draw a neat diagram to illustrate the data flow. |  |
| b) | Draw a timing diagram of MVI B,80H instruction. |  |
| c) | Explain the following: i) Any two rotate instructions ii) Any two branching instructions iii) Any two stack related instructions. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a program to find the number of 1’s in a given data byte stored at memory location 9050H. |  |
| b) | Write a program to read 3 bytes of data starting from memory location 8050H. Complement each byte and store the result at successive memory locations starting at 9010H. |  |
| c) | Write a program to find the largest of 10 numbers stored in memory locations starting at 2000H. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail the 8085 interrupt process. |  |
| b) | Explain the concept of nesting of subroutines. Explain any four conditional return instructions based on four different flags. |  |
| c) | What are Restart instructions? Explain in detail. |  |
| Q.No.6. | Write short note on any four: | 4 x 3 = 12 |
| a) | Looping and indexing |  |
| b) | Interfacing of keyboards and displays |  |
| c) | Memory mapping |  |
| d) | RIM and SIM instruction |  |
| e) | Time Delay routines |  |
| f) | Conditional CALL and RET instructions |  |

***********