## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg./P

Subject: Machine Design Practice(4045)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the following in brief: i) Endurance limit ii) Fatigue failure iii) Creep. |  |
| b) | Define Factor of Safety. What are the general considerations for selection of Factor of safety? |  |
| c) | With the help of neat sketch explain: i) Single transverse fillet weld ii) Double transverse fillet weld. |  |
| d) | What is coupling? What are the requirements of good coupling? |  |
| e) | State three points of difference between ‘self locking screw’ and ‘overhauling screw’. |  |
| f) | Define the following for Helical spring: i) Spring Index ii) Pitch iii) Solid length. |  |
| g) | How will you select bearing from Manufacturer’s catalogue? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A socket and spigot cotter joint is subjected to an axial load of 30KN. Design the following parts of the Joint: i) Rod ii) Spigot rod end iii) cotter. The material used for all the parts are carbon steel for which following allowable stress may be used. Tensile stress t =50MPa; Shear stress s =35 MPa; Crushing stress c =90 MPa. |  |
| b) | The Pull in the rod of an Iron roof truss is 50KN. Design the following part of Adjustable Turn Buckle to sustain the pull: i)Tie rod ii) Coupler Nut (length and outside diameter). Assume the following stresses for both the parts: t = 75 N/mm2; s = 37.5 N/mm2; c = 90 N/mm2. |  |
| c) | A suspension link shown in Fig.1, is subjected to a maximum load of 150 KN. The permissible stresses in tension, shearing and crushing are 100MPa, 75 MPa and150 MPa respectively. The width of each side link is 60 mm. Design the suspension link. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A solid shaft transmits a power of 20 KW at 400 rpm. Find the diameter of shaft on the basis of i) strength ii) Rigidity. State which diameter you will adopt. Assume the following for shaft Material: Torsional shear stress : 50N/mm2 Modulus of Rigidity : 0.8 x 105N/mm2 Permissible angle of Twist : 0.4 per meter length Length of shaft : 10 m |  |
| b) | Fig. No.2 shows a shaft with Pulley at one end. The Pulley is 200mm in diameter, weighs 300N and is driven by a belt which is vertical. The angle of wrap is 180 and the coefficient of friction between belt and Pulley is 0.3. If the Maximum power transmitted is 1KW at 500 rpm, determine the diameter of shaft. Assume the allowable shear stress of the shaft material as 50MPa. |  |
| c) | Fig. No.3 shows a C- clamp which a carries of a load ‘P’ of 25KN. The cross section of the clamp is rectangular, the depth to width ratio b/t is 2:1. If the permissible tensile stress is 100N/mm2, determine the dimensions of ‘b’ and ‘t’. |  |
| Q.No.4. | Answer the following Questions: | 2 x 6 = 12 |
| a) | A bolt is subjected to an axial shock load so that the maximum applied load equals to 12 KN. Assuming an allowable tensile stress of 50N/mm2, determine the diameter of bolt using fine threads. Neglect stress concentration. It is desired to increase the shock absorbing capacity of the above bolt. Determine the diameter of the hole required to be drilled in the bolt to make it a Bolt of uniform strength. |  |
| b) | An ISA 200 x 100 x 10 angle is welded to a steel plate by means of fillet welds as shown in Fig. No.4. The Angle is subjected to a static force of 150KN. If the permissible shear stress for weld material is 70MPa, determine the lengths of weld ‘l1’ and ‘l2’ as shown in the figure. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a Muff coupling used for connecting two shafts transmitting 30KW at 400rpm. The material for the shaft and key is plain carbon steel for which the maximum shear stress and crushing stress are 100MPa and 200MPa respectively. The Muff is of cast Iron for which maximum shear stress is 30MPa. Use factor of safety as ‘2’. |  |
| b) | The screw of a shaft straightener has single start square threads of 70mm outside diameter and 10mm pitch. It exerts a load of 60KN. The coefficient of friction between screw and nut is 0.12. Determine the following: i) Effort required at the end of handle 600mm long ii) Total bearing pressure on the threads. |  |
|  |  | ……..3/- |
|  | -3- |  |
| c) | i) Explain in brief the various failures that can occur in sunk key. |  |
|  | ii) State four advantages and two limitations of Deep groove ball bearing. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A helical spring has a stiffness 90 N/mm, and has its end squared and ground. The permissible shear stress for the spring material is 600MPa. The load on the spring causes a deflection of 8.5mm. Determine the following: i) Diameter of spring wire ii) Active number of coils iii) Total number of coils iv) Free length for spring. Assume Spring Index C=6, Modulus of Rigidity = 0.84x 105N/mm2, Wahl’s factor K=(4C−1)(4C−4)+0.615C. |  |
| b) | A semi elliptical leaf spring has the following specifications: Effective distance between the eyes = 600 mm Central load = 3 KN Central deflection = 15 mm Permissible bending stress = 135 N/mm2 Width to thickness ratio b/t =12 Modulus of elasticity of spring material E= 2x105 N/mm2 Determine i) size of leaves ii) Number of leaves. |  |
| c) | i)What is Proof stress? Explain how the Proof stress is determined? |  |
|  | ii)State three advantages and three limitations of rolling contact bearing over sliding contact bearing. |  |