## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Automobile Engg./P

Subject: Machine Tools & Production Process-II(4060)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the purpose of having a quick return mechanism in a shaper? |  |
| b) | What is angular milling? |  |
| c) | Explain how T-bolts and clamps are used in drilling operations. |  |
| d) | The risk of injuries and accidents is highest when using the Planer in machine shops. Why? |  |
| e) | What are the safety precautions to be taken while working on a Slotter? |  |
| f) | Describe a fly cutter used in boring machines with a neat sketch. |  |
| g) | What is (i) Side milling and (ii) Face millng? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note on tool holding devices of a drilling machine. Explain why ‘Morse taper’ is provided in these devices and its value. |  |
| b) | Describe any two work holding devices used in drilling. |  |
| c) | Enlist the various operations performed on a drilling machine and explain any one in detail. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and label a horizontal boring machine. |  |
| b) | Describe with a neat sketch any one type of vertical boring machine. |  |
| c) | How are Shapers classified? Give a brief description of each type. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe any one quick return mechanism used in a Shaper. |  |
| b) | In a Shaper work, the various parameters are: |  |
|  | -Strokes per minute = 50 (double) |  |
|  | -Quick return ratio= 1:2.5 |  |
|  | -Work piece dimensions =500mm(L) x 30mm (B) |  |
|  | - Feed =2 mm per stroke (double) |  |
|  | Find the cutting speed and total machining time, if clearance stroke is neglected. |  |
| c) | Write notes on: |  |
|  | (i) Machining flat surfaces in a Slotter |  |
|  | (ii) Machining key ways in a Slotter |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw two views of a double housing planner and label its different parts. |  |
| b) | Enumerate various milling machine operations and explain any two with the help of neat sketches. |  |
| c) | Define with respect to a milling machine: |  |
|  | (i) Cutting speed (ii) Feed (iii) Depth of out. | ………2/- |

-2-

| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Classification of milling cutters |  |
| b) | A reversible electric motor drive used in a Planer |  |
| c) | Slotter tools |  |
| d) | Up-milling and Down-milling |  |
| e) | Gang Drilling and Multi spindle drilling |  |

***********