## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg.

Subject: Fluid Machinery(4040)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | A jet of water 35mm in diameter having a velocity of 25m/s, strikes normally a flat smooth plate. Determine the thrust on the plate, if the plate is at rest. |  |
| b) | What do you understand by the term minimum starting speed? Write the mathematical equation for minimum starting speed. |  |
| c) | What is the condition for hydraulic efficiency of a Pelton wheel to be maximum? |  |
| d) | What is negative slip in reciprocating pump? |  |
| e) | What is the function of shuttle valve? Write its symbol. |  |
| f) | How is unloading valve useful in hydraulic circuits? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Pelton wheel has a mean bucket speed of 10m/s with a jet of water flowing at a rate of 0.7m3/s under a head of 30m. The buckets deflect the jet through an angle of 160. Calculate the power and efficiency of the turbine. Assume coefficient of velocity as 0.98. |  |
| b) | What is cavitation? On what factors does the cavitation in water turbine depend? Enumerate some methods to avoid cavitation in water turbine. |  |
| c) | With a help of a neat sketch describe multi-stage pump with (i) impellers in series and (ii) impellers in parallel. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Francis turbine of 135KW,having an overall efficiency of 75%, is required to produce under a head of 9m and at 120rpm. The velocity at the periphery of the wheel is 3.47H and the velocity of flow is 1.1H. If the hydraulic losses in the turbine are 20% of the available energy, find: (i) guide blade angle at inlet (ii) wheel vane angle at inlet (iii) diameter of the wheel. |  |
| b) | A centrifugal pump delivers water against a head of 14.5m while running at 1000 rpm. The vanes are curved back at an angle of 30 with the periphery. The impeller diameter is 300mm and outlet width 50mm. If manometric efficiency of the pump is 85% find the discharge of the pump. |  |
| c) | Define indicator diagram. Draw an indicator diagram, considering the effect of acceleration and friction in suction and delivery pipes. Write the mathematical expression for work done per second in case of a single-acting reciprocating pump. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What is the significance of characteristics curves for centrifugal pumps? Draw the following characteristic curve for the centrifugal pump: Head, power and efficiency versus discharge with constant speed. |  |
| b) | With the help of a neat sketch explain the governing mechanism of a reaction turbine. |  |
| c) | A single-acting reciprocating pump, running at 60rpm, delivers 0.53m3 of water per minute. The diameter of the piston is 200mm and stroke length 300mm. The suction and delivery heads are 4m and 12m respectively. Determine theoretical discharge, coefficient of discharge and power required to run the pump. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the construction and working of a reciprocating pump. |  |
| b) | Explain the construction and working of a rotary vane pump with the help of a neat sketch. |  |
| c) | Distinguish between an internal gear pump and external gear pump. Explain any one with the help of a neat sketch. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the construction and working of a safety valve. |  |
| b) | Describe a Regenerative type of a hydraulic circuit, with the help of a neat sketch. |  |
| c) | Explain meter-out type of hydraulic circuit with the help of a neat sketch. |  |

***********