## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Theory of Structures-II(4023)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | A three hinged arch of span 8m and rise 1.5m carries an UDL of 20KN/m for left span and a point load of 30KN at 2 m from right end. Determine reaction at supports. |  |
| b) | A column of 40 x 60mm has 1.5m length with both ends hinged. Find Buckling load by Euler’s formula. Take E=200 KN/mm2. |  |
| c) | A fixed beam of span 4m has UDL of 20 KN/m for ½ span length. Find Fixed End Moments. |  |
| d) | Define i) Carry Over Factor ii) Relative stiffness and Distribution Factor. |  |
| e) | A simply supported beam of 150 x 200mm in section is of span 6m with UDL of 20KN/m throughout. Determine maximum deflection in beam. E = 0.14 x 105 N/mm2. |  |
| Q.No.2. | Answer any one of the following Questions: | (12) |
| a) | A simply supported beam of 6m span carries point loads of 40 KN at 2m, 60KN at 4m from left end of beam. Find slope at both supports and deflection under both loads. Ixx = 34620 cm4, E =2 x 105 N/mm2. |  |
| b) | A propped cantilever of 4m span carries UDL of 30 KN/m throughout and a point load of 20KN at mid span. Determine reactions at prop and sketch SFD and BMD. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Fixed beam of 4 m span carries a point load of 40 KN at 1m from left. Find FEMs and sketch SFD and BMD. |  |
| b) | A Fixed beam of span 6m carries UDL of 40 KN/m for a length of 4m. Find FEMs and sketch BMD of beam. |  |
| c) | A Fixed beam of 5m carries point loads of 20KN and 30KN at 2m and 3m from left. Find FEMs and sketch SFD of beam. |  |
| Q.No.4. | Answer any one of the following Questions: | (12) |
| a) | Analyse a continuous beam ABC with AB=5m and BC=5m. ‘A’ has a fixed support while ‘C’ is a simple support. The span AB carries UDL of 35KN/m throughout while BC has point load of 25KN at 2m from end C. Assume EI uniform throughout. Draw SFD and BMD. |  |
| b) | A portal frame ABCD with height of column AB=CD=6m and beam BC=7m. UDL of 50 KN/m acts on BC. The flexural regidity (EI) of column is 2EI and of beam is 1.5EI. Supports A and D are fixed. Analyse frame and sketch BMD by Moment Distribution method. |  |

………2/-

-2-

| Q.No.5. | Answer any one of the following Questions: | (12) |
| --- | --- | --- |
| a) | A hollow mild steel column of 220mm outer diameter and 160mm internal diameter with height of column as 6m. One end of column is fixed while the other is hinged. Find safe load carried by column by using i) Euler’s method with E=2 x 105 N/mm2 and Factor of safety = 2.5 ii) Rankine’s method with  = 1/7500 and crushing stress as 320 MPa and Factor of safety = 3. |  |
| b) | A principal rafter of a truss consists of two angle sections ISA 150x150x12 placed on either side of gusset plate of 12mm thick. Determine Buckling load by Rankine’s formula. Take crushing stress as 300N/mm2 and = 1/7000. The properties of single angle section are: A =3477mm2, Ixx = Iyy = 746.3 x 104mm4, Cxx=Cyy=41.6mm. |  |
| Q.No.6. | Answer any one of the following Questions: | (12) |
| a) | A three hinged arch of 16m span with central rise 4m with both supports at same level. Point loads of 60KN, 80KN and 60 KN acts at distances of 2m, 6m and 10m from left end. Find i) Reaction at supports ii) Normal thrust and Radial shear at 4m from left end. |  |
| b) | A three hinged arch of 40m span with central rise 6m. An UDL of 120 KN/m acts on left half span. Find i) Reaction at supports ii) Maximum bending moment in the arch in magnitude and location of same. |  |

***********