## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Automobile Engg.

Subject: Hydraulic Machinery(4053)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the terms i) Compressibility ii) Specific weight. |  |
| b) | Define barometric pressure, guage pressure and absolute pressure. |  |
| c) | State and explain Pascal’s Law. |  |
| d) | State and explain Bernoulli’s theorem. |  |
| e) | Explain the phenomenon of water hammer. |  |
| f) | What is priming of centrifugal pump? State different methods of priming. |  |
| g) | What are the functions of air vessels in reciprocating pump? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A circular plate of diameter 1.2m is placed vertically in water in such a way that the centre of the plate is 2.5m below the free surface of water. Determine i) Total pressure on the plate ii) Position of centre of pressure. |  |
| b) | A horizontal venturimeter with inlet diameter 200mm and throat diameter 100mm is used to measure the flow of water. The pressure at inlet is 0.18 N/mm2 and the vacuum pressure at the throat is 280mm of mercury. Find the rate of flow. The value of Cd may be taken as 0.98. |  |
| c) | With the help of a neat sketch, explain the working of a Micromanometer. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the discharge through a trapezoidal notch which is 1.2m wide at the top and 0.50m wide at the bottom and is 0.4m in height. The head of water on the notch is 0.3m. Assume cd for rectangular portion = 0.62, while for triangular portion = 0.60. |  |
| b) | A vertical sharp-edged orifice 120mm in diameter is discharging water at the rate of 98.2 litres/s under a constant head of 10 metres. A point on the jet, measured from the vena contracta of the jet has co-ordinates 4.5 metres horizontal and 0.54 metre vertical. Find the following for the orifice: i) Coefficient of velocity ii) Coefficient of discharge iii) Coefficient of contraction |  |
| c) | With the help of a neat sketch, explain the working of a hydraulic accumulator. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of suitable examples, explain i) Hydraulic gradient line ii) Total energy line. |  |
| b) | With the help of a neat sketch, explain the construction and working of a centrifugal pump. |  |
| c) | Define i) Slip ii) Negative slip iii) Separation in reciprocating pump |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With the help of a neat sketch, explain the construction and working of an external gear pump. |  |
| b) | With the help of a neat sketch, explain the construction and working of a hydraulic intensifier. |  |
| c) | State six differences between centrifugal pump and reciprocating pump. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | State 2 applications each of i) Vane pump ii) Plunger pump. |  |
| b) | State the laws of fluid friction. |  |
| c) | Draw sketches of any 2 types of impellers used in centrifugal pump. |  |
| d) | Define i) Dynamic Viscosity ii) Kinematic Viscocity. |  |

***********