## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Civil Engg./CT

Subject: Quantity Surveying & Costing(4014)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Why is it necessary to prepare estimate in construction Industry? |  |
| b) | Explain the terms i) Centage ii) Provisional sums iii) spot items. |  |
| c) | What are functions and mode of preparing an abstract? |  |
| d) | What is task work? How is it worked out for different items in building construction? |  |
| e) | State factors affecting the value of an asset. |  |
| f) | Write advantages and disadvantages of piecework method of executing work. |  |
| g) | Mention circumstances under which lowest tender can be rejected. |  |
| h) | What is purpose of writing specifications? |  |
| Q.No.2. | Answer any one of the following Questions: | (15) |
| a) | A plan and wall section of a single storeyed structure is shown in Fig.No.1. Work out the quantities of any five of the following items: - P.C.C. 1:3:6 in foundation and plinth - Laterite stone masonry in foundation and plinth in CM 1:6 - Brick masonry in superstructure 30cm thick in CM 1:6 - R.C.C slab 1:1.5:3, 12 cm thick - Internal plaster 1:4 CM, 12mm thick - Ceramic tile flooring - Woodwork in door/window shutters. |  |
| b) | Using suitable method estimate quantity of earthwork for a road between two stations A to B with following data. One chain =30m, width of road=12m, side slopes in cutting 1.5:1, side slopes in banking 2:1. Draw longitudinal and cross-sectional view. Chainage; 0; 1; 2; 3; 4; 5; 6; 7. RL of Ground (m); 124.70; 124.90; 124.60; 122.90; 121.60; 121.00; 120.40; 120.00. RL of Formation (m); 122.20; 122.40; 122.60; 122.80; 123.00; 123.20; 123.40; 123.60. Assume any suitable data, if required. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Explain how will you carry out approximate estimate for G+1, 3 bedroom residential house with built up area of 140 sq. metres. |  |
| b) | Carry out rate analysis of laterite stone masonry in CM 1:4 for 10m3 quantity of work. |  |
| c) | Carry out rate analysis for R.C.C 1:1.5:3 including centering and shuttering but excluding steelwork for 10m3 quantity of work. | ……2/- |
|  | -2- |  |
| d) | Explain in brief valuation and rent fixation of a Government building. |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Draft a tender notice inviting tenders for construction of a government school building at the estimate cost of Rs.1.2 crores to be completed in 20 months, executed by the PWD. |  |
| b) | Classify the works executed by PWD and explain briefly each of them. |  |
| c) | Write a short note on cash imprest and temporary advance. |  |
| d) | Differentiate between lump sum contract and item rate contract. |  |
| Q.No.5. | Write short notes on any five: | 5 x 3 = 15 |
| a) | Revised Estimate V/s Supplementary Estimate |  |
| b) | Contract document |  |
| c) | Schedule of Rates |  |
| d) | Book value V/s Market value |  |
| e) | Earnest money deposit and security deposits |  |
| f) | Types of specifications |  |
| g) | Arbitration |  |