## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electronics Engg./EC

Subject: Communication Systems-I(4150)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Noise figure ii) Signal to noise ratio. |  |
| b) | Define modulation & explain the need for modulation. |  |
| c) | Compare PM & FM. |  |
| d) | Draw frequency spectrum of stereo FM modulated signal. |  |
| e) | Explain the working principle of Superheterodyne Receiver. |  |
| f) | Explain the function of Amplitude limiter in FM receiver. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain cause & effects of various types of external noises. |  |
| b) | Explain the generation of AM using Grid modulation technique. |  |
| c) | A 500 KHz, 6V carrier is amplitude modulated by a 4V modulating signal having maximum frequency of 10 KHz. Calculate: i) Modulation index ii) Frequency & amplitude of sidebands iii) Bandwidth iv) Total power, Carrier power, Side band power, if antenna resistance is 100. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the method of suppressing the carrier signal from an AM signal using neat circuit diagram & necessary derivations. |  |
| b) | Explain the concept of VSB technique of transmission using neat diagram. |  |
| c) | Define FM. State the mathematical representation of FM wave & modulation index, with the help of neat waveforms. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State & explain the advantages of FM over AM. |  |
| b) | Explain Armstrong method of generating FM with neat block diagram & vector diagrams. |  |
| c) | Explain the block diagram operation of High Power level AM transmitter. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of practical diode detector circuit. |  |
| b) | Explain the operation of Balanced slope detector circuit. Also state the limitations of the same. |  |
| c) | Explain sensitivity & selectivity of a Radio receiver in detail. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the need of AGC in a Radio Receiver. Also explain the concept of delayed AGC in communication receiver. |  |
| b) | With the help of block diagram, explain the operation of Stereophonic FM receiver. |  |
| c) | Explain phase shift method of suppressing the carrier. |  |

***********