## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg.

Subject: Principles of Airconditioning(5047)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the following Psychrometric properties: (i) Dew point temperature (ii) Relative humidity (iii) Saturated air. |  |
| b) | List three heat sources which contribute to Internal Heat gain of an A/c space. |  |
| c) | What are the impurities present in outside air? |  |
| d) | Classify supply air outlets on the basis of their location. |  |
| e) | What are the factors to be considered for selection and location of Return Inlets? |  |
| f) | What are the factors which affect the thermal capacity of an Insulating material? |  |
| g) | Explain in brief types of Insulating materials. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A sample of moist air is at 35C DBT and 10C dew point temperature under a total pressure of 1.013 bar. Using steam table and emperical formulae, determine the following: (i) Partial pressure of vapour (ii) Specific humidity (iii) Relative humidity. |  |
| b) | With the help of neat sketch, explain the construction and working of “Aspirating Psychrometer”. |  |
| c) | A duct carries 360m3/min of air at a velocity of 240m/min. Determine the diameter of duct using friction chart. Determine the equivalent square cross section, assuming the velocity in both the ducts same. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following Psychrometric process and represent the process on Psychrometric chart: (i) Heating and de-humidifying (ii) Cooling and humidifying. |  |
| b) | What is meant by By-pass air? Derive an expression for By-pass factor of a heating coil. |  |
| c) | With the help of a neat block diagram, explain Air conditioning system for hot and dry summer condition. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled block diagram of Central Air conditioning–DX system. |  |
| b) | Explain i) All-water system ii) Air-water system. |  |
| c) | Discuss special features of air conditioning of hotels and restaurants. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the method of estimating the cooling load of an Air-conditioned space with the help of Cooling Load Estimation chart. |  |
|  |  | ……2/- |
|  | -2- |  |
| b) | Following Data refers to the conditioning of an office. - Occupancy load (sensible) – 6000 KJ/min - Occupancy load (Latent) - 4200 KJ/min - Lighting Load (Fluorescent) - 1200 KJ/min - Solar Heat gain (Glass) - 3600 KJ/min - Solar Heat gain (Wall & Roof)- 4200 KJ/min - Outside air heat (sensible) - 4800 KJ/min - Outside air heat (Latent) - 2400 KJ/min. Determine i) RSH ii) RLH iii) OASH iv) OALH v) GSH vi) GLH vii) GTH viii) RSHF. |  |
| c) | Explain with a neat sketch, chemical de-humidification process. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch, explain the construction and working of ‘Electric Air-filter’. |  |
| b) | With a neat sketch, explain two-position electrical control system. |  |
| c) | Explain any two types of humidity control elements. |  |

***********