## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Automobile Engg.

Subject: Thermal Engineering(4052) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the greatest source of heat loss in a boiler plant? How does an economiser help in reducing it? |  |
| b) | Define i) Dryness-fraction ii) Degree of superheat of steam. |  |
| c) | Give three advantages of steam turbine over a reciprocating engine. |  |
| d) | State the use of compressed air. |  |
| e) | Define i) Emissivity ii) Absorptivity of a substance. |  |
| f) | List five desirable properties of an ideal refrigerant. |  |
| g) | Compare the relative advantages and disadvantages of 4-stroke and 2-stroke cycle I.C engines. |  |
| h) | List the functions of steam condenser in steam power plant. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the construction and working of any one safety mounting. |  |
| b) | With the help of a neat sketch explain the working of a Locomotive boiler. |  |
| c) | Following observation were made during a test on a steam boiler: Quantity of coal burnt per hour 750 kg Feed water supplied per hour 7000kg at 25c. Calorific value of coal 33910 KJ/Kg Feed water temperature leaving economiser 80c Dryness fraction of steam leaving the boiler 0.95 Heated temperature of steam 25C Determine thermal efficiency of the plant. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Steam is at a pressure of 10 bars. If the dryness fraction of steam is 0.8, obtain the latent heat and enthalpy of steam. |  |
| b) | List the classification of steam turbines based on different criteria. |  |
| c) | With the help of a neat sketch explain how dryness-fraction of steam is determined by using a combined separating and throttling calorimeter. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the working of 4-stroke I.C. engine. |  |
| b) | An engine consumes 8kg of oil per hour, the calorific value of oil is 30,000 KJ/Kg. If the brake power of the engine is 22KW and the mechanical efficiency is 85%, calculate the following: - Indicated Power - Indicate thermal efficiency - Brake thermal efficiency. |  |
| c) | Draw a neat diagram showing two-stage reciprocating compressor with inter-cooler. Explain the advantages of intercooling. | ……2/- |

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A tube has internal radius 20mm, external radius 25mm. The inside of the tube is maintained at 100C and the outside at 20C. Calculate the quality of heat conducted through unit length of tube. Take K=380 w/mk. |  |
| b) | Distinguish between Jet condenser and Surface condenser. |  |
| c) | Define Logarithmic Mean Temperature Difference (LMTD) and explain the concept of Black body. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is a cooling tower? Give its functions and describe Natural-draught, Forced-draught and Induced draught cooling tower. |  |
| b) | Give the advantages and disadvantages of an Air-Refrigeration system |  |
| c) | Explain the working principle of Vapour Compression Refrigeration cycle. |  |

***********