## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: Power Generation, Transmission & Distribution(EE304) [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State any three advantages of diesel power plant. |  |
| b) | Briefly explain why conductors are transposed. |  |
| c) | What is skin effect? Briefly explain. |  |
| d) | State any three advantages of ring main system. |  |
| e) | List the factors affecting earth resistance. |  |
| f) | State any three I.E. rules relevant to light and fan wiring. |  |
| g) | Give permissible voltage drops in various parts of distribution system for urban, semi urban and rural areas. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain merits of inter connected grid system. Also state its demerits. |  |
| b) | With the help of schematic diagram explain nuclear power station. |  |
| c) | A generating station supplies following loads: 15,000KW, 12,000 KW, 8,500 KW, 6,000 KW & 450 KW. The station has maximum demand of 22,000 KW. The annual load factor is 48%. Calculate: - The number of units supplied annually - The diversity factor - The demand factor. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State requirement of line supports. State features, advantages and disadvantages of steel poles. |  |
| b) | Compare pin type and suspension type Insulator. |  |
| c) | Describe and state advantages and disadvantages of - Copper conductor (ii) ASCR conductor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is corona? Explain how corona is formed. |  |
| b) | Explain HVDC system. State its advantages. |  |
| c) | The potential across the 6 units of the string is equalized by using graded insulators. If the capacitance of top insulator is 8C and that of pin of earth is C. calculate capacitance of other units. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain classification of distribution system w.r.t voltage and number of wires. Briefly explain AC, 3 wire system of distribution. |  |
| b) | Explain radial type of distribution and state its advantages and disadvantages. |  |
| c) | With neat diagram explain plate earthing. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | Briefly explain cleat wiring. | (3) |
|  | ### OR |  |
| a) | Draw and explain wiring diagram for godown wiring. | (3) |
| b) | A classroom of 4m x 4m x 3m requires 4 fans and 6 tube lights: (i) Draw a plan showing suitable location for fan and tubes (ii) Estimate total electrical wiring material. (assume PVC casing and capping type of wiring) | (9) |

***********