## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Machines-I(EE302) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Compare between lap and wave winding for a DC machine. (Any 3 points) |  |
| b) | Explain the classification of DC generators with respect to connection of its field and armature winding. |  |
| c) | What is function of a DC motor? Briefly explain its working principle. |  |
| d) | Explain the concept and significance of back emf in the operation of DC motor. |  |
| e) | Briefly explain the necessity of tap changer in transformer. List the types of tap changers used in transformers. |  |
| f) | Distinguish between power and distribution transformer. (Any 3 points) |  |
| g) | Briefly explain the concept of open-delta connected transformer with neat diagram. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List various parts of DC machine. State the functions of any four of its parts. |  |
| b) | Explain the process of voltage buildup in a self excited dc shunt generator by drawing the relevant characteristics. State any two conditions necessary for its successful voltage buildup. |  |
| c) | Explain the necessity of parallel operation of dc generators. State any two conditions necessary for satisfactory parallel operation of dc shunt generators. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of equation of speed, list the various speed control methods for dc motor. Explain any one of these methods for dc shunt motor, with a neat diagram. |  |
| b) | With a neat diagram explain the construction and operation for a 4 point starter for dc shunt motor. |  |
| c) | Explain the concept of ‘magnetic leakage flux’ in a transformer. State its effects on the operation of transformer and its methods adopted for reducing it. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the necessity of cooling in a transformer? List the various methods of cooling adopted for transformer cooling. Explain any one of them. |  |
| b) | The core of 100KVA 11KV/440V, 50Hz, 1 core type transformer has cross section of 20cm x 15cm. Find the following: 1) Number of HV and LV winding turns 2) Emf per turn if the maximum flux density is 1.4 Tesla 3) Current through HV and LV winding. |  |

……..2/-

- 2 -

| c) | Explain the no load and on load operation of a 1 practical transformer having secondary current with lagging power factor. Draw the relevant phasor diagram. |  |
| --- | --- | --- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 50KVA, 4400/220V, 1, 50Hz transformer has R1=3.45, R2=0.009, X1 = 5.2 and X2 = 0.015. Calculate the following: 1) Ro1 2) Ro2 3) Xo1 4) Xo2 5) Zo1 & 6) Zo2. |  |
| b) | The readings obtained form O.C. & S.C. tests performed on a 10KVA , 440V/120V, 50Hz, 1 transformer are as under: O.C. test: Vo=120V; Io=4.2A; Wo= 80W  Readings obtained on LV winding. S.C. test:Vsc=9.65V; Isc=22.2A;Wsc=120Wwith LV winding short circuited. Compute the percentage voltage regulation and percentage efficiency for a load with 0.8 lagging p.f. |  |
| c) | Explain the concept and significance of voltage regulation in a transformer. Write its equation for leading and lagging p.f. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List advantages offered by the parallel operation of two transformers. Explain the procedure for parallel operation of two 1 transformers. State any two necessary conditions for the same. |  |
| b) | With a neat diagram explain the working principle of an autotransformer. List its advantages over a two winding transformer. Also list its applications. |  |
| c) | Write short note on following: 1) Furnace transformer 2) Welding transformer. |  |

***********