## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-I(4001) [Old Course]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What scope does the subject of surveying has in the field of Civil engineering? |  |
| b) | List the instruments used for Ranging with brief description. |  |
| c) | Explain the following terms: (i) Base line (ii) Check line (iii) Oblique offset. |  |
| d) | What is meant by open and closed traversing? Give example. |  |
| e) | Mention the specific use of following survey minor instruments: (i) Sextant (ii) Planimeter (iii) Clinometer. |  |
| f) | What are the sources of errors in levelling? Mention the precautions to be taken. |  |
| g) | List the methods of reduction of levels. Which method is most appropriate for a rural survey? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the difference between ‘direct’ and ‘indirect’ ranging. When each one can be practiced? |  |
| b) | How will you overcome the following obstacles in chaining: - A river is crossing the chainage line - A building is obstructing the vision and chaining. |  |
| c) | Explain the use of following instruments for setting out perpendicular to a survey line: (i) Optical square (ii) Open cross staff. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between the following: - Fore bearing and Back bearing - Whole circle Bearing and Reduced bearing - Magnetic bearing and true bearing. |  |
| b) | The following bearings were observed in a closed traverse. Find the included angles and correct bearing of lines. Draw sketch of the plot. LINE; FB; BB. AB; 140 45′; 318 15′. BC; 216 30′; 38 0′. CD; 209 15′; 30 15′. DE; 319 45′; 139 45′. EA; 60 15′; 240 15′. |  |
| c) | The following bearings were observed in a closed compass traverse: LINE; FB; BB. AB; 305 0′; 125 30′. BC; 75 30′; 254 30′. CD; 115 30′; 297 0′. DE; 165 30′; 345 30′. EA; 225 0′; 44 0′. At which of these stations would local attraction be suspected? Find the corrected bearings of these lines. | ……2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain traversing by included angle and the graphical adjustment of closing error. |  |
| b) | Explain the following terms: - Datum surface - Reduced level - Temporary Bench mark - GTS Bench mark - Line of collimation - Axis of bubble tube |  |
| c) | The staff readings for a survey work were as follows: 1.810, 2.110,1.225,1.455, 0.905, 2.435, 2.810, 2.675 and 1.765. The level was shifted over the 4th and 7th readings. The first R.L was taken on a B.M. whose RL is 100.000. Rule out a page of level book and enter the readings. - Work out the RL’s of all stations. - Apply arithmetical check. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the essential difference between ‘Temporary adjustment’ and ‘Permanent adjustment’ of a dumpy level. Explain any one in brief. |  |
| b) | The following reading was taken on a survey line with back sight being underlined. 0.813, 2.170, 2.908, 2.630, 3.133, 3.752, 3.277, 1.899, 2.390, 2.810, 1.542, 1.274, 0.643. The first reading was taken on a Bench mark of R.L 39.563. Enter the readings in a level book form and find all the R.Ls of points. Apply usual check. |  |
| c) | Draw a neat sketch of ‘Abney level’ and explain it’s primary use in the field. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | General principles of survey |  |
| b) | Method of stepping on a sloping ground |  |
| c) | Precaution in selection of Triangulation stations |  |
| d) | Cross section levelling |  |

***********