## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg./E/FT

Subject: Refrigeration & Airconditioning(5034)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Psychrometric Charts is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Coefficient of performance. |  |
| b) | Write the designations of following refrigerants: |  |
|  | i) Monochloro di Fluoro methane |  |
|  | ii) Carbon dioxide |  |
| c) | State important properties of refrigerant Ammonia. |  |
| d) | Draw a neat sketch of a P-H diagram, when refrigerant is superheated before compression and under-cooled before expansion for a vapour compression system. |  |
| e) | Draw a neat sketch of any Psychrometer and briefly explain its function. |  |
| f) | What are the requirements of good room air distribution? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cooling system works between –7C and 27C. The vapour is dry at the end of adiabatic compression. There is no under cooling. Determine the C.O.P. of the vapour–compression system. Temp C; Enthalpy KJ/Kg; Entropy KJ/KgK. Liquid; Vapour; Liquid; Vapour. -7; -30; 1268; -0.108; 4.642. 27; 115; 1288; 4.27; 4.33. |  |
| b) | With a neat sketch explain the construction and working of a three– fluid vapour absorption system. |  |
| c) | What is the function of duct system? Enumerate duct systems and explain any one of them with a neat sketch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the construction and working of a thermostatic expansion valve. |  |
| b) | Describe with a neat sketch window airconditioner. |  |
| c) | The atmospheric air has 35C dry bulb temperature and 50% R.H. Using psychrometric chart, find: |  |
|  | (i) Wet bulb depression (ii) Dew point depression. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | 2 Kg of air at 15C dry bulb temperature and 13C wet bulb temperature is mixed with 1Kg of air at 25C dry bulb temperature and 50% RH. Determine the dry bulb temperature and wet bulb temperature of the resulting mixture. |  |
| b) | Explain with a neat sketch, construction and working of a ice plant. |  |
| c) | What is the function of Air washer? With a neat sketch explain construction and working of air washer. |  |
|  |  | ……..2/- |
|  | -2- |  |
| Q.No.5. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Temperature–concentration diagram |  |
| b) | Screw compressor |  |
| c) | Finned tube type Evaporator |  |
| d) | Psychrometric process of cooling and dehumidification |  |
| e) | Automatic humidity control |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Cascade system |  |
| b) | Water cooled condenser |  |
| c) | Room sensible heat factor |  |
| d) | Limit switches and time switches |  |
| e) | Duct design |  |

***********