## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-I(4037)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Steam tables and Mollier’s charts is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State and explain second law of thermodynamics. |  |
| b) | What is meant by the internal energy of steam? |  |
| c) | State the uses of compressed air. |  |
| d) | Draw T-S diagram of modified Rankine cycle and write equation for its efficiency. |  |
| e) | Distinguish between ‘characteristics Gas Constant’ and ‘Universal Gas constant’. |  |
| f) | What is the significance of dryness fraction of steam? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A sample of coal has the following composition by mass: Carbon 75%, hydrogen 6%, oxygen 8%, nitrogen 2.5%, sulphur 1.5% and ash 7%. Calculate its higher and lower calorific values per kg of coal. Assume: Latent heat of vaporization of water = 2466 KJ/Kg. |  |
| b) | With a neat sketch explain the construction and working of Boy’s gas calorimeter. |  |
| c) | Define boiler. Differentiate between water-tube boiler and fire tube boiler. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define and represent the following processes on P-V and T-S diagram: i) Adiabatic process ii) Isobaric process. |  |
| b) | A quantity of air has a volume of 0.028 m3 at a pressure of 1.25 bar and 25ºC. It is compressed to a volume of 0.0042m3 according to the law pv1.3 = constant. Find the final temperature and work done during compression. |  |
| c) | With the help of a neat sketch, explain the construction and working of Benson high pressure boiler. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the internal energy of 1 kg of superheated steam at a pressure of 10 bar and 280ºC. Assume specific heat of superheated steam as 2.1 KJ/KgK. |  |
| b) | When is multi-stage compression used for air? Explain the effect of intercooling in a a multi-stage reciprocating compressor. |  |
| c) | With the help of a neat sketch, explain the construction and working of fusible plug. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Dry saturated steam at a pressure of 10 bar is expanded isentropically to a pressure of 0.7 bar. Using Mollier diagram, determine the enthalpy drop. |  |
| b) | A steam engine receives dry and saturated steam at 10 bar pressure and exhausts at 0.1 bar. Determine the Rankine efficiency. | ……2/- |
|  | -2- |  |
| c) | State the function of a steam calorimeter. Explain the construction and working of any steam calorimeter with a neat sketch. |  |
| Q.No.6. | Write short note on any three: | 3 x 4 = 12 |
| a) | Geothermal Energy |  |
| b) | Axial flow air compressor |  |
| c) | Boiler draught |  |
| d) | Air motor |  |
| e) | Limitations of fossil fuels as energy source |  |

***********