## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical Engg./E&EE

Subject: Transformers(4106)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between core and shell type transformer. |  |
| b) | Explain the construction and functions of a conservator in a transformer. |  |
| c) | With a neat sketch, explain in brief distributed crossover type of transformer’s winding. |  |
| d) | What are the advantages and disadvantages of autotransformers? |  |
| e) | Enlist applications of autotransformers. |  |
| f) | Explain in brief furnace transformer with a special features. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Deduce the emf equation of a transformer & state the factors on which it depends. |  |
| b) | With a neat sketch explain the construction and operation of an OFF load tap changer. |  |
| c) | State the different types of cooling systems employed in a transformer and explain the ONAN method of cooling in detail. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) What are the different advantages of per unit system? |  |
|  | ii) Draw and explain the phasor diagram of a transformer on NO load. |  |
| b) | A single phase transformer has 1200 turns on primary and 300 turns in secondary side. The No load current is 2.5A and the no load power factor is 0.2 lagging. Calculate the primary current and its power factor when the secondary draw a current of 300A at 0.8 power factor lagging. |  |
| c) | A 30KVA, 2400/120V, 50Hz transformer has a high voltage winding reactance of 0.1 ohm and a leakage reactance of 0.22 ohm. The low voltage winding resistance is 0.035 ohm and the leakage reactance is 0.012 ohm. Find the equivalent winding resistance, reactance and impedance referred to the i) high voltage side and ii) Low voltage side. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define efficiency of a transformer. Derive the condition for maximum efficiency. |  |
| b) | A 10KVA single phase transformers, rated for 2000/400V has resistance and leakage reactance as follows: Primary winding: R1=5.5 ohms, X1=12 ohms. Secondary winding: R2=0.2 ohms, X2 = 0.45 ohms. Determine the approximately value of secondary voltage and voltage regulation at full load, 0.8 p.f lagging when the primary voltage is 2000V. |  |
|  |  | …….2/- |
|  | -2- |  |
| c) | A 100KVA distribution transformer has full load losses of 4 KW. The losses being equally divided between core and copper losses. During the 24 hours, transformer is loaded as follows: 6am to 10am (4 hours) – half load 10am to 6pm (6 hours) - 1/4th full load 6pm to 10pm (4 hours) – full load 10pm to 6am (8 hours) - negligible load Calculate the all day efficiency of the transformer. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the conditions that must be fulfilled for the successful parallel operation of single and three phase transformers. |  |
| b) | Explain the four vector groups pertaining to 3-phase transformers. Is it possible to operate transformers of different vector groups in parallel? Justify your answer. |  |
| c) | Two 3-phase transformers A & B having same no load line voltage ratio 3300V/400V supply a load of 750 KVA and 0.707 lagging. When operating in parallel, the rating of transformer A is 500KVA, resistance is 2% and reactance is 3%. The corresponding values of B are 250 KVA, 1.5% & 4% respectively. Assuming both transformers are having star connected secondary windings. Calculate (i) load shared by each transformer & (ii) p.f at which each transformer is working and secondary line voltage of the parallel circuit. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the phasor diagram of a transformer supplying a resistive load. |  |
| b) | Find the efficiency of a 50KVA transformer at ¾ full load at 0.8 p.f lagging. The copper losses at full load are 1500W and iron losses are 1300 W. Ignore the effect of temperature and magnetising current. |  |
| c) | Explain the construction and characteristics of a welding transformer. |  |

***********