## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-II(4335)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Answer any six Questions including Q.No.1 which is compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Explain any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Purpose of Inclining Experiment |  |
| b) | Permeability and use of floodable length |  |
| c) | Bale capacity and grain capacity |  |
| d) | Side launching |  |
| e) | Freeboard and its significance |  |
| f) | Weight curve |  |
| g) | Margin line |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | Describe the load, shear force and bending moment diagram of ship in still water. Explain how these diagrams are drawn. | (8) |
| b) | Explain how section modulus of a ship is obtained. | (4) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Explain the procedure to carry out GRT and NRT calculations. | (8) |
| b) | What is freeboard and why is it provided on ships? | (4) |
| Q.No.4. | A vessel is loaded up ready for departure. KM is 11.9m and KG is 9.52m with a displacement of 20550 tonnes. From the ship’s cross curves of stability, the GZ ordinates for a displacement of 20550 tonnes and a VCG of 8m above base are as follows: Angle of heel (); 0; 15; 30; 45; 60; 75; 90. GZ ordinate; 0; 1.10; 2.22; 2.60; 2.21; 1.25; 0.36. Using the information, construct the ships for this condition of loading and determine the following: - Maximum righting lever GZ - Angles of heel at which this maximum GZ occurs - Range of stability. | (12) |
| Q.No.5. | A box shaped vessel 100 meters long x 20 meters wide x 12 meters deep is floating in salt water on an even keel at 6 metres draft. A forward compartment 8 metres long, 20m wide and containing cargo of permeability 25 percent is bilged. Find the new drafts after flooding. | (12) |
| Q.No.6. | Describe with the sketch launching curves used in stern launching of vessel. What are the important features of the curves? | (12) |
| Q.No.7. | A box shaped barge of 80m length and lightship weight of 520 tonnes is divided by transverse bulkheads into 4 equal holds. Each hold is loaded uniformly with following cargo: Hold No. 1; 540 tonnes. Hold No. 2; 460 tonnes. Hold No. 3; 520 tonnes. Hold No. 4; 520 tonnes. Draw the SF and BM diagram indicating maximum bending moment. | (12) |

***********