## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg.

Subject: Welding Engineering(5041)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain in brief the machines commonly used for cutting metals in workshop. |  |
| b) | Sketch some of the basic symbols used to represent welds. |  |
| c) | Define TIG welding process. |  |
| d) | What is Arc blow? Which are the types of arc blow? |  |
| e) | Explain resistance spot welding cycle. |  |
| f) | How do cracks and blowholes occur in welded joints? |  |
| g) | Compare weld inspection and weld testing. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the metallurgical effects caused due to welding operations. |  |
| b) | With a neat sketch, explain the two methods of gas welding. |  |
| c) | What is blow-pipe? Explain the two types of blow pipes. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain submerged Arc welding process with a neat sketch. |  |
| b) | Discuss welding cables and electrode holder of arc welding. |  |
| c) | Explain the causes for distortion in welded structure. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the advantages and disadvantages of Friction welding. |  |
| b) | Explain Thermit welding process with a neat sketch. |  |
| c) | List the various safety precautions to be followed while welding in confined spaces. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss incomplete penetration and slag inclusion as defects in weldment. |  |
| b) | Compare X-rays and Gamma rays radiography test. |  |
| c) | With a neat sketch explain Brinell and Rockwell hardness test. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Classification of welding processes |  |
| b) | Welding positions |  |
| c) | Types of distortion |  |
| d) | Backfire |  |

***********