## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg.

Subject: Machine Drawing-II(4035)

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any four of the following Questions: | 4 x 3 = 12 |
| --- | --- | --- |
| a) | Calculate the limits of size for the shaft 35g6 using the following data: Diameter step: 30mm to 40mm Magnitude of tolerance, IT6 =10i i=0.453D+0.001D, where D=D1×D2 (i is in microns, D is in mm) Fundamental deviation = -9 microns |  |
| b) | Indicate the roughness grade symbols for the following roughness grade numbers: N11, N7, N4, N1. |  |
| c) | Explain the meaning of the geometrical tolerances indicated in microns for the machine component shown in fig. No.1. |  |
| d) | Show how the roughness is indicated on the component for the following situations: i) Surface to be chrome plated ii) Surface to be obtained without removal of material iii) Surface to be obtained by shaping. |  |
| e) | Draw the following pipe fittings in double line: i) Globe valve ii) Lateral. |  |
| f) | Draw the symbols for the following welds: i) Single – V butt weld ii) Double – J butt weld iii) Plug weld iv) Seam weld. |  |
| Q.No.2. | Draw neat proportionate free hand sketches of any two of the following: | 2 x 6 = 12 |
| a) b) c) | Gib and cotter joint – Sectional front view, sideview. Oldham’s coupling – Sectional front view, side view Socket and spigot pipe joint – front view with top half in section, side view. |  |
| Q.No.3. | Answer any one of the following Questions: | (15) |
| a) | Fig.No.2 shows the details of the gland and stuffing box. Draw to scale the following views of the assembly using First angle method of projection: - Sectional front view - (10) - Top view - (5) | ……2/- |
|  | -2- |  |
| b) | Fig. No.3 shows the front view with top half in section and right side view of the assembly of the simple eccentric. Draw to scale the following: - Sheave –front view and top view - (8) - Strap ‘A’ – front view and top view - (7) Mention all the dimensions. |  |
| Q.No.4. | Fig. No4 shows the orthographic views of the V-belt pulley. Redraw the views and incorporate the following details: - End faces of the rim to be imparted surface roughness indicated by . - Roughness on surface of hole of 25mm to be indicated by . - Tolerance on bore of 25mm to be indicated as H7. - Axis of hole of 25mm to be indicated as datum A. - Radial run-out not to exceed 0.1mm on surface of rim of 250 with reference to datum A. - Left end face of the rim to be parallel to right end face within 0.02mm. | (12) |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Fig.No.5 shows the orthographic views of a machine component fabricated by welding. Redraw the given views and indicate the appropriate welding symbols. |  |
| b) | Fig. No.6 shows the single line orthographic views of a piping layout. Draw the single line isometric view of the same. Mention all the dimensions. |  |
| Q.No.6. | Study the working drawing of the flange of a protected type flange coupling shown in fig. No.7 and answer any eight of the following: - What is the maximum diameter of the flange? - What is the outer diameter of the hub? - What is the thickness of the circumferential flange? - How many holes of 16mm are provided? - What is the pitch circle diameter of 16mm holes? - What is the taper of the keyway? - What is the tolerance indicated on the hole of 50mm? - What is the width of the keyway? - What is the surface roughness indicated on the right end face of the flange? x) What is the use of the projection(spigot) of 75mm and 5mm thickness? | (12) |

…….3/-