## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Architectural Engg.

Subject: Building Construction-III(AE303)[Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Why is spacer bar used in grillage foundation? |  |
| b) | State the ratio adapted for designing of RCC slab as one-way or two-way slab. |  |
| c) | Draw a detail sketch of timber handrail fixed to a M.S. baluster. |  |
| d) | Draw & label different sections adapted to fabricate M.S. grills in a small residential bungalow. |  |
| e) | State any three materials used as roof covering over timber roof. |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | A hall measuring 20m x 10m with framed structure has to be constructed on a flat land. Make suitable assumptions & explain in detail procedure for setting of foundation trenches. | (6) |
|  | ### OR |  |
| a) | Explain in detail procedure to cast a simple isolated footing. Make suitable assumptions. | (6) |
| b) | State the condition under which following types of foundations could be suggested: i) Grillage foundation ii) Combined footing. | (3) |
| c) | Draw a detail of holdfast fixed to wooden window frame. | (3) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a detailed note on ‘scaffolding’. |  |
| b) | Write a descriptive note on methods of forming construction joint. |  |
| c) | Explain the procedure followed for construction of RCC column from plinth level upto floor slab. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain different ways of spanning of R.C.C staircases with neat sketches. |  |
| b) | Write a note on classification of stairs based on shapes. |  |
| c) | Design & draw a steel stair to climb a mezzanine floor of a shop. Assume suitable height of mezzanine floor. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat labelled elevation two leaf panelled door for a residential house. |  |
| b) | Explain various ways of fixing M.S. grill to a window 1.50m x 1.50m. |  |
| c) | Draw various types of joints commonly adopted in construction of panelled door. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw single line sketches of any three types of trusses. |  |
| b) | Write a short note on single roof & its types. |  |
| c) | Draw a neat sketch to explain fixing of A.C. sheet over a timber roof. |  |

***********