## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Process-II(4236)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What should be the clearance between the shearing blades? How the blades are arranged? Sketch & briefly explain. |  |
| b) | List the advantages of Rotary Shearing Machine. |  |
| c) | What are the safety precautions to be observed in Cropping & Notching Machines? |  |
| d) | How do you specify the size of the centre lathe? |  |
| e) | List the different types of Milling Machines. |  |
| f) | Which are the operations that are carried out on Shaping Machines? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch differentiate between “UP Milling” & “Down Milling”. |  |
| b) | What are the differences between Shaper & Planer? (Any six points) |  |
| c) | List the various types of grinding wheels. How is the grinding wheel specified? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the various types of grinding machines & sketch any two types of rough grinders. |  |
| b) | A 50mm thick plate is required to be cut (straight line) by Oxy-Acetylene flame. Explain the step by step method along with equipment required. |  |
| c) | Write short note on: (i) Profile cutting Machine (ii) Template allowance. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of sketch explain the principle of working of Plasma Arc Cutting. |  |
| b) | Write short note on: (i) Pipe end Bevelling Machine (ii) Plate Bevelling Machine. |  |
| c) | Draw a neat sketch of horizontal boring machine & explain its working. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of sketch explain the difference between “Counter Shunk hole” & “Counter bored hole”. |  |
| b) | List the various methods of taper turning on lathe & sketch & explain any one method. |  |
| c) | Write short note on: (i) Safety precautions while working on guillotine machine (ii) Bench shearing machine. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Uses of Presses in straightening |  |
| b) | Types of Rotary Shearing Machine |  |
| c) | Shear type nibbler. |  |
| d) | Work holding devices on Shaper |  |
| e) | Punch type nibbler |  |
| f) | Tools used for straightening |  |

***********