## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electrical Engg./EE/COMP/EC/MED/E&EE/EI

Subject: Digital Electronics(CS309) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give the symbol, Truth table and expression for a two input EXOR Gate. |  |
| b) | Convert the following Gray code to equivalent binary numbers: i) 1011 ii) 10110 iii) 11100111. |  |
| c) | Name the three methods used for Analog to Digital conversion. |  |
| d) | Give the block diagram of a 4 bit parallel Adder. |  |
| e) | Compare Asynchronous and Synchronous counters. |  |
| f) | Give the diagram of 4 bit SISO register. |  |
| g) | Convert the following: - 56 = _______________(16) - 27(16) = _______________ (2) - 10111(2) = _______________(10) |  |
| h) | Draw the circuit and write the Truth table of R-S flipflop using NAND Gates. |  |
| i) | Represent the following Numbers in BCD form: i) 26 ii) 39 iii) 145. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is Multiplexer? Draw the logic diagram of a 4:1 Multiplexer using basic logic gates. Explain the operation with the help of Truth Table. |  |
| b) | Construct OR, NOT, AND gates using universal NOR Gate. Give the Truth Table for each. |  |
| c) | - Prove the following using Boolean Algebra A+B+C = (A+B) (A+C) - Simplify using K-Maps Y=Σm (1, 3, 4, 5, 7, 9, 10, 11, 12, 13, 14, 15) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of a clocked R-S flip flop using NAND Gates with the help of a neat diagram and Truth Table. |  |
| b) | Give the symbol of JK flip flop. Explain its operation along with the Truth Table. |  |
| c) | Write short notes on: (i) D flip flop (ii) T flip flop. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the diagram of 4 bit shift register and explain how the data 1011 is shifted serially IN and parallely taken out. |  |
| b) | Draw the logic diagram of 4 bit Asynchronous UP counter. Write the count sequence and show the timing diagram of the output w.r.t clock pulses. |  |
| c) | What is the decade counter? Draw the logic diagram of a Decade counter. Give the count sequence. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Draw the diagram of successive approximation converter and explain the operation. What are its advantages? |  |
| b) | With a neat diagram, explain the working of 4 bit Binary ladder type D/A converter. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Encoder |  |
| b) | Ring Counter |  |
| c) | ASCII Code |  |
| d) | Binary Addition |  |
| e) | Full Subtractor |  |

***********