## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mechanical Engg./P/A

Subject: Machine Drawing-I(4033)  [Old Scheme]

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Attempt any two of the following: | 2 x 6 =12 |
| --- | --- | --- |
| a) | A line AB has its end A 15 mm above HP and 10mm infront of VP, end B is 40mm above HP and 35 mm infront of VP. The distance between its end projectors is 46mm. Draw the projections of the line and find out its true length. Use auxillary plane method. |  |
| b) | A Semi circular lamina of diameter 80mm is resting on its diametral edge in HP. Draw its projection when this edge makes an angle of 35 to VP and lamina makes an angle of 45 to HP. Draw by change of ground line method. |  |
| c) | A triangular pyramid having base equilateral triangle of side 40mm and having height 55 mm is resting in HP with respect to one of the edge of the base. Triangular face containing this edge perpendicular to HP. Draw its projections when this edge of the base is inclined at 40 to VP in such a way that its triangular face is visible to the observer. Use auxillary method of projection. |  |
| Q.No.2. | Answer the following: |  |
| a) | A regular hexagonal pyramid, edge of base 20mm and height 60mm rests on its base with one on its base edge perpendicular to VP. An auxillary inclined plane inclined to HP at 30 and perpendiculars to the VP cuts its axis at a distance of 30mm from the base. Develop the lateral surface of the truncated prism. | (10) |
|  | ### OR |  |
| b) | A right regular square prism of base 30mm edge and 60mm height, rests on its base on HP such that its vertical faces are equally inclined to VP. It has a horizontal circular hole of 30mm diameter drilled centrally through it such that the axis of the hole cuts both the diagonally opposite vertical edges. Develop the lateral surface of the prism. | (10) |
| Q.No.3. | Answer the following: |  |
| a) | A Square prism, edge of base 44 mm and height 72mm rests on its base on ground with one of its faces inclined at 60 to VP. It has a hole of diameter 50mm drilled centrally through it. The centre line of the hole is parallel to both the HP and the VP and is 5mm infront of the axis of the prism. Draw the projections of solid showing the curves of intersection. | (12) |
|  | ### OR |  |
| b) | A cone of 90mm height and 55mm diameter of base is resting on ground with its axis perpendicular to the ground. A horizontal cylinder of 40mm diameter and height 80mm is penetrated by the cone apex completely. Draw the projections and show the curves of intersection. | (12) |

………2/-

-2-

| Q.No.4. | Fig.1 shows the front view and top view in 1st angle method of projection. Draw according to 3rd angle method of projection: i) Sectional front view along AA ii) Sectional Top view along BB iii) Right hand side view. | (16) |
| --- | --- | --- |
| Q.No.5. | Answer the following: |  |
| a) | Give conventional representation for the following: - Glass - Bend gear - Wood - Tension spring. | (08) |
|  | ### OR |  |
| a) | With the help of a suitable freehand sketch show the following: - Full section - Removed Section - Offset Section - Revolved Section. | (08) |
| b) | Draw by double line method a 25mm diameter pipe showing: - Gate valve - Reducing Elbow - Elbow 45. | (09) |
| Q.No.6. | Answer the following: |  |
| a) | The object shown in fig.2 is to be fabricated by welding at different pieces A, B, C and D in the proper relative positions. Draw the orthographic view of the object and indicate the proper welding symbols. | (08) |
|  | ### OR |  |
| b) | Show by means of sketches the method of showing location, symbol, size and depth of the following forms of weld: 1) Single V-butt weld 2) Single bevel butt weld 3) All round fillet weld. | (08) |