## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electronics & Communication Engg./EE

Subject: Data Communication(5138)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Why are uplink and downlink frequencies different is satellite communication? |  |
| b) | What are passive satellites? List two applications of passive satellites. |  |
| c) | Describe the synchronous communication format. |  |
| d) | What is the need for the transport layer in the ISO–OSI model? |  |
| e) | Describe a graded–index optical fibre. |  |
| f) | What are the advantages of using optical communication? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a diagram, describe the construction of a LASER diode and its principle of operation. |  |
| b) | Describe the principle of operation of a PIN diode with the help of a diagram. |  |
| c) | Explain multimode propagation in an optical fibre. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the functions of the physical and network layers of the ISO–OSI protocol. |  |
| b) | What are U–frames and why is a U–frame required in the HDLC protocol? Describe the U–frame format. |  |
| c) | Describe the construction of typical fibre optic cable. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the BISYNC protocol used in computer communications. |  |
| b) | Explain with the help of diagrams the ring and star topologies. |  |
| c) | Explain the concept of circuit switching and list two differences between circuit and packet switching. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the block diagram of the receiver subsystem of an earth station and describe it. |  |
| b) | Why is station–keeping required in satellite communication? How is it done? |  |
| c) | Describe the telemetry, tracking and control subsystem of a satellite. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | List two differences between: - Synchronous and asynchronous communication - Mesh and bus topologies. |  |
| b) | Write a short note on INSAT. |  |
| c) | Describe four applications of satellite communication. |  |
| d) | Describe any one coupling method used with optical fibres. |  |

***********