## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Electronics Engg./EC/EI/COMP/MED

Subject: Microprocessors(PD&I)-II(4147)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write the control word needed to initialise an 8255 for port A – handshake input, port B – handshake output, port C – PC6 and PC7 as outputs. |  |
| b) | Explain the concept of DMA data transfer. |  |
| c) | Describe the keyboard/display mode set control word of 8279. |  |
| d) | Explain the function of the following registers in 8259: i) IRR ii) IMR iii) ISR. |  |
| e) | Explain any 3 timer modes of the 8155. |  |
| f) | List the main features of RS 232 Bus standard. |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Write a program to read the DIP switches interfaced to port A of 8255 and display the data on LEDs interfaced to port B. Draw a schematic for the interfacing. |  |
| b) | Explain the sequence of events in data output from a 8155 port to a peripheral. Draw the timing waveforms. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the block diagram of Programmable Communication Interface IC 8251. |  |
| b) | Describe the 8253 control word format. |  |
| c) | Draw and describe the keyboard section of 8279. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the features of 8259 Programmable Interrupt Controller. |  |
| b) | Describe the 8259 initialization command words ICW1 and ICW2. |  |
| c) | Draw and explain the block diagram of 8257 DMA controller. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain the working of a laser printer. |  |
| b) | Explain the working of any two key switches. |  |
| c) | Explain the function of the bus management lines of the GPIB. |  |
| Q.No.6. | Write notes on the following: | 2 x 6 = 12 |
| a) | Hard disk |  |
| b) | Temperature controller |  |

***********