## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2014 Examinations

Programme: Mining Engg.

Subject: Surface Mining(4298)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between long term planning & short term planning. |  |
| b) | What is a Plant Mixed Slurry (PMS)? Give its advantages. |  |
| c) | What are the regulations pertaining to steep workings? |  |
| d) | Give the applications of a belt conveyor. |  |
| e) | How do you stabilize the dump slopes? |  |
| f) | Write short notes on chamber blasting. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the principle of ripping. What are the guidelines to be followed while ripping? |  |
| b) | Explain the methods or techniques of drilling and give their application. |  |
| c) | Write short notes on i) Controlled blasting ii) Deck blasting. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the parameters in overcasting by shovel. |  |
| b) | What are the applications of a dragline? Explain operation of the bucket of a dragline. |  |
| c) | How do you find the capacity of a bucket wheel excavator? Write a brief note on operation of bucket wheel. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the different methods of reclamation. |  |
| b) | Explain the factors affecting the bench parameters. |  |
| c) | Write short notes on i) stripping ratio ii) pit limits. |  |
| Q.No.5. | Answer the following |  |
| a) | i) Calculate the number of holes required to blast 5000T of coal in a 7m height bench with a burden of units. Assume other data. | (7) |
|  | ii) Calculate the output of an excavator with 3.5 cu-m bucket & a cycle time of 35 seconds. Assume other data if required. |  |
| b) | Explain the layers & the construction material used in haul road. | (5) |
|  | ### OR |  |
| b) | Differentiate between the operation of the different types of dumpers. | (5) |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Why wheel loaders are popular in Goan mines? |  |
| b) | What are the different types of dumps & give their features? |  |
| c) | What are the types of belts & their application? |  |
| d) | How do you evaluate the production efficiency of dozer? |  |

***********